The intheboatshed.net Julie skiff

15ft 6in length overall by 4ft beam, by 450lbs displacement and 33.5sq ft wetted area, designed for stitch and glue construction using 4-6mm 4ft by 8ft plywood

This boat has been designed by an amateur with no qualifications in boat design or boatbuilding and should be regarded as experimental until several have been built. The designer accepts no liability for any loss or accident that may result from following these instructions or their attendant plans or from any loss or accident that may follow from using the boat.

- Design points

The Julie skiff as laid out in these plans is a lightweight stitch and glue flattie skiff that would be a nice rower for one and can carry two at a pinch. The name came from my partner, who took rather a shine to the boat.

It's designed easy to build and use, to be a pleasure to row, easy on the eye, and to be dry and comfortable. I've drawn what I consider to be a very attractive sheerline, a moderately large transom that should keep out following waves, and a bows intended to lift over oncoming swells.

Another feature designed for comfort is the sunken foredeck, which drains back into the water and not into the boat. It's intended to provide a convenient space to put potentially wet and mucky things such as a painter or a small anchor and rode, and will also redirect splashes back into the water. If anyone wishes to sit at the front of this little boat, they'll have to sit on the bottom, but I don't think that will be any great loss.

On the lines, I have to say that I've never known a small boat design that defined itself as this one did. In fact, within the confines of the beam, length and displacement there weren't too many options for making a new flattie skiff that was radically different or better than many of the existing designs. Nevertheless, the Julie skiff is all my own work and does not share her lines or construction with any other existing skiff design that I know about. Any similarities with other boats that may be found by others are purely coincidental.

My approach has been to make the forward lines fairly straight, while making those aft more curvy in order to place the centre of bouyancy in the conventional place, a little aft of the half way mark. This approach seems to have worked well with another design of mine, the Light Trow, which is another light-displacement flattie design of about this size and is reported to row very well.

I've also varied the flare - the angle between the sides of the boat and its bottom - throughout the length of the boat. In the forward quarter of the boat flare has been used to create a shape that will turn back small waves, while at the centre of the boat it is reduced in order to work in the necessary displacement.

Further aft, however, it has been allowed to increase again in an attempt to reduce the potential for the hard chine to create eddies, and also to ensure that the stern has plenty of bouyancy if the user wishes to transport something heavy - or someone heavy - in the stern.

- Construction issues

Readers may with to read my book Ultrasimple Boatbuilding published by International Marine and available from high street and online bookstores.

Before going any further, it's important to check the size of the plywood available to you, as it comes in different sizes in different countries around the world. A sheet of ply in the UK and the USA is likely to be 4ft (1220mm) wide, and 8ft (2440mm) or a little more in length, and this is the size these plans are made for. However, in the rest of the world including Australia, New Zealand an continental Europe ply sizes can vary considerably.

I'd like also to emphasise the importance of model-making. The download package for this boat includes drawings intended to be used to make a model, and I'd ask you to please make a model first as it will show you first how the boat will look but perhaps more importantly how each component part goes together and contributes it's pennyworth towards creating a usable boat with a rigid structure.

The system of coordinates I use to define the shapes of the panels on the 4 by 8ft plywood panels is the aspect of the plans most likely to cause fear, uncertainty and doubt. This is  simply because the approach seems unfamiliar, but in fact the technique is the same as the one most of us used to draw graphs when we were at school - though I'd argue that building a boat is a darn site more fun than maths lessons.

Each pair of coordinates describes a particular point on a plywood sheet. The first number (the X-coordinate) in each pair of coordinates is a measurement along the bottom of the sheet, while the second in the pair (the Y coordinate) is a measurement up from the bottom - that is along a line that is a right angle (or 90 degrees) from our point along the bottom.

Before plotting each of these coordinates one by one, I find it best to square off the material into twelve-inch squares, as it's so much easier to plot from your squaring off than it is to mark out each measurement from the panel edge, but also because it helps to show whether you material is really 8ft long.

Ply is frequently a little longer than it's supposed to be, and if you don't remove the extra before starting to build the boat, you could be in trouble, with some components longer than others.

For squaring off ply, I often like to use a drywall square, but a tape measure and the side and edge of another sheet of plywood will do the job perfectly well, if slightly less conveniently.

Once the lines have been drawn, you need to draw your cutting lines. I do this by first finding a flexible batten to help re-create create the curves, and I've found that perhaps the best thing to use is an 8ft plastic moulding of the kind one can buy in any do-it-yourself story.

To make the curves, drive small nails into the material at each of the plotted points, lay the batten along the nails, using weights to hold it up to the nails. When the batten is in place and reasonably secure, draw along the length of the cutting line using a soft carpenter's pencil. After a few hours of plotting and drawing, you should be able to see that the lines on your ply correspond to those on my plans.

A final word of warning on marking out. There are a lot of coordinates to plot here, and that means there are a lot of opportunities to make mistakes, both for the builder and for the designer. If when you have plotted the points and drawn the lines there are any that don't look right, check them, and check them again. Remember the old carpenter's advice that one should measure two or three times before cutting once. If after all this checking you're in doubt that I may have made an error please contact me at gmatkin@clara.net - I don't want you to risk wasting material unnecessarily because of a mistake I have made.

The next job is to cut the material out, and to build the boat using the stitch and glue boatbuilding method, otherwise known as tack and tape construction.

Anyone who has used this method before will see immediately how it will work with this design, but if any of you reading this haven't worked this way before I won't explain it here but I would recommend reading it up at almost any epoxy supplier's website. One example is the UK Epoxy website: http://www.epoxy-resins.co.uk/.

I'd make only three general construction points.

First, these drawings have been created with the intention that the fore and aft sections of the bottom and sides should be joined by epoxy tape on either side of the join, rather than using butt blocks or a scarf. I often prefer this method for several reasons: it's easy, it allows me to get the longest boat possible from two sheets of ply, and unlike using butt blocks, I don't afterwards have to cut the thickness of the butt block from any frames they happen to coincide with. If you do build this boat using butt blocks, you'll no doubt have to cut some material from the central frames to make up.

Second, the bows of this lightweight boat have no special reinforcing. This is because builders working in stitch and glue are expected and advised to reinforce the inside of the bows with several layers of epoxy and tape instead of the usual single layer. I'd suggest applying a single layer of tape, then two further layers laid each to either side, and finally a fourth and perhaps a fifth layer of tape over the joint on the centreline. Each layer of tape should be extended onto the bottom of the boat to make a smooth and strong joint, with each layer of tape extend s little further aft. Of course, if you'd rather make a false stem to fit on the inside of the bows, that's fine with the designer, but please don't forget to include a knee against the bottom and foredeck.

Third, once the sides are bent around the frames, you'll find that the sides become slightly curved where they meed the frames, and that there will be a small gap between the frame and the side-plank. That's only to be expected from the geometry of the side planks. All you have to do is to make sure this gap gets well filled with epoxy and covered over with tape in the usual way. The joint will be as strong or stronger than a similar joint with no small gap.

There are a several special construction features of this boat that require a little explanation.

Foredeck scuppers There are two drains or scuppers on each side of the boat at the aft end of the foredeck. These aren't absolutely essential but I think they should be cut after the boat has been assembled in order to prevent water accumulating in the sunken foredeck. The holes for the drains should be 2-3in in length by 1 in from top to bottom.

Inwales and frames I'm very much in favour of gapped inwales, which can be made to look as they appear in the drawings - they make a boat very rigid, and they look good. A typical gapped inwale might be made up of a 1.5 deep by 1in wide inwale (I'd make these up from two 1 by 1/2in laths laminated together) on 1in square spacers at 12-16in distances; of course, you'll have to adjust your spacing and notch the frames so that the inwales fit nicely into them.

However, in a rowing boat built to this design they may not be strictly necessary, so long as you enlarge the gunwales to make up for the loss. I'd suggest laminating two lengths of 1 by 1 1/2in material to make a gunwale that's close to two inches wide. If you decide to go without the inwale, you'll find that the frames I've drawn leave a hard corner that you will want to round off.

Breasthook and quarter knees If you build with inwales, you'll need to cut them down to accept the breasthook and the quarter knees, but if you build with a heavier external gunwale, I'd suggest they can be attached by simple stitch and glue. The breasthook and quarter knees I've drawn here represent the minimum, by the way – if you'd like to make them larger and more elegant, perhaps with a curved form, go ahead but try not to cut into the minimum area I've defined.

Limber holes I've drawn large-ish limber holes in the centre of each frame. These are optional and I'd probably prefer to seal up these areas, fit drain plugs or rotating hatches in pairs and use them for built in bouyancy and dry storage. However, others will prefer to include the limber holes in order to keep rot to a minimum, and yet others working with stitch and glue construction may prefer to site them at the chines, so long at they're using 5-6mm ply. The limber holes I've drawn will also be particularly useful to those builders who decide to build this boat from material lighter than the 5-6mm the designer envisaged, as it will allow them to build in a keelson for most of the length of the boat.

Doubled stern Sterns have to be strong and I'd prefer to double this part of the boat. However if this boat is build in 5-6mm ply, is to be rowed only (that is, not sculled, sailed or driven using an outboard), and won't ever be used for towing another boat, the double-thickness stern may be dispensed with.

Positioning of oarlocks I haven't drawn a position for the oarlocks as this can be a matter of taste. The standard is that they should be about 12in aft of the aft edge of the transom, but some argue this distance should be the length of the rower's forearm, and this may be particularly important for taller rowers. Oarlocks should be mounted on solid blocks of 8-12in in length, 5in in depth and 2in thick or so. They will have to be shaped to match the curve of the boat, and if you are using gapped inwales they should be cut so that they fit into the inwale and can be glued to it (with the gapped inwale installed first, naturally).

Thwart If there is anywhere on this boat where a piece of good quality timber with nicely shamfered or rounded edges, this may be it. Think of the hours you're looking forward to spending on it!

Oar length There are a lot of views about oar length. Some say the length of an oar should be of half the beam (from lock to lock) times three, plus 6 inches (6ft 6in), others that the length of an oar from lock to grip should be half the beam times three plus 2 inches (6ft 2in), still others use much more complicated calculations based on the speed of the boat and how lightly it rows. However, I'd say about 7ft will be perfectly acceptable where this boat is concerned.

Trim line In some of the drawings I've included a piece of 1/2in material 5in below the lower edge of the  gunwale, starting the same distance back from the bows and running all the way to the stern. I'd suggest it makes an elegant decoration, but adds little to the structure of the boat!

Foot brace If the boat is to be used as a tender, a foot brace might get in the way, but if it is to be used for pleasure rowing, then I'd suggest knocking up a a comb and using something like the drawing below: